- Consider a host with the following information:
IP Address. . . . . . . .  : 131.183.116.98
Subnet Mask . . . . . . : 255.255.224.0
Default Gateway . . . : 131.183.127.252

- 1) What Class is this address?

- 2) How would this IP address be expressed using CIDR notation?

- 3) What portion of this IP address identifies the network?

- 4) What portion of this IP address identifies the host?

- 5) Has it been sub-netted? How do you know?

- 6) Does the “network” use a fixed length subnet mask (FSLM) or a variable length subnet mask! (VLSM)?

7) An organization wanted to connect 3 remote sites (Findlay, Fostoria, Fremont) to the corporate site.

- The IP network available for the remote sites is: 206.244.21.0/24.

- The Findlay site has 64 workstations on the LAN. There are 16 workstations in Fostoria, and 34workstations in Fremont.

- Propose a subnet design for the 3 sites.

8) An organization has an Internet connection and has been assigned the IP address 201.201.201.0/24.

- The router connecting the corporation to the Internet has been assigned an IP address of 201.201.201.1/24.

- There is a LAN at the main corporate site consisting of 49 hosts.

- Within the next 3 months, the company  plans to open an office in Perrysburg with 22 hosts and a third office in Delta with 16 hosts.

- The company plans to increase the number of hosts on the corporate LAN by 49 within 1 year.

- There is potential for doubling the size of the remote  offices within 6 to 9 months.

- Unfortunately, the IP address space available to the corporation is limited to a single Class C.

- Will a single Class C be sufficient for this organization? Explain