Alexander J King
The University of Toledo College of Engineering

CSET4750 Computer Networks and Data Communications

LAB 8: Network Design/ Computer Ethics

1) Acme Computing Corp. has remote sites in Port Clinton and Napoleon in addition to their main site in Toledo. There are currently 29 devices connected to the network in Toledo. The corporate plan is to link the Port Clinton and Napoleon offices to Toledo via T-1 lines. The remote offices which are due to open in the next 4 weeks will have a small number (less than 14) of devices connected to a LAN in each office. The corporation has obtained Cisco routers to deploy at the remote locations and a Cisco for use in Toledo (www.cisco.com). In the near future, Acme plans to connect to the Internet and Toledo will serve as the connection point. Propose a sub-network design that will meet the needs of Acme Corporation. For the first implementation of the plan, assume that we will use unnumbered connections between the routers and that the routing protocol will use static IP addresses. Identify any initial issues that you can foresee.

What are we doing? → subnetting → designing a network.

#1 priority is Scalability – ensure it can scale because we do not want to add devices 6 months from now and run out of IP addresses in a group and have to do it all again. Build it to last! 
 2 sub questions
 1. Do we want to use public or private addresses?  (public costs money! / private is free)
 2. Do we want to use VLS or fixed length subnet masks?  Variable are much more of a pain. Not as easy to do some things. Fixed length is easy and we prefer it.

If we’re using public we use variable length to save money. If we use private we just load em up fixed length. Balance between wasting money and longevity.

Create IP addressing scheme. Something standardized. EX: we could have all of our access points be formed like: 192.168.0.1 / 192.168.1.1 / 192.168.2.1 / 192.168.3.1 etc….. 192.168.groupnumber.routeris1

Small places use 192.168 large places will use 10.

I initially wanted to start with a /24 and split it into 4 groups of 64 addresses. 0-63,64-127, 128-192, 192-256

1,64,128,192 would be the routers because it is easy to recall.

We have to live with these choices long term so it might be a better plan to make it more scalable. 
This would mean instead of having locations be 192.168.0.x 1,64,128,192 being the location it might be easier to recall addresses and easier to scale if I pick to do it 192.168.0.x, 192.168.1.x, 192.168.2.x, 192.168.3.x etc for location / devices.

So I want 4 subnet groups as described above .0.1, .1.1, .2.1, .3.1. This will allow 256 addresses per location. More growth and easier to identify location and device makes it the better choice in my opinion after reviewing this in class. 

Static IP addresses can be nice because they’re more secure and change as infrequently as possible. The downsides are having to manually enter new devices every time one arrives or departs. The positive is you know the ip address of the printer, routers, or switches for example and it’s never going to change hopefully.

2) What are the Ten Commandments of Computer Ethics? Who/what created them? Why were they created?

1. Thou shalt not use a computer to harm other people.

2. Thou shalt not interfere with other people's computer work.

3. Thou shalt not snoop around in other people's computer files.

4. Thou shalt not use a computer to steal.

5. Thou shalt not use a computer to bear false witness.

6. Thou shalt not copy or use proprietary software for which you have not paid.

♪♫ don’t copy that floppy♪♫

7. Thou shalt not use other people's computer resources without authorization or proper compensation.

8. Thou shalt not appropriate other people's intellectual output.

9. Thou shalt think about the social consequences of the program you are writing or the system you are designing.

10. Thou shalt always use a computer in ways that ensure consideration and respect for your fellow humans.

Computer ethics institute. Used to educate.

3) What are the procedures to follow in diagnosing network problems?

Check and double check to ensure it is not a problem on our end. Then contact kindly and courteously the location where we think the problem lies.

4) You have followed the procedures’ in question 3 above and have diagnosed a problem to lie with equipment not under your administration. How do you handle the situation.

Call the person who administer the equipment with a conscious effort to be kind and courteous. If we’re going to be delayed many hours or even days we need to triple and quadruple check the issues is not on our end.